How to Laser-cut a Scanned Sketch.

Step 1.

- Open Corel Draw

- Import your scanned image:  File > Import > image.jpeg

Step 2.

Now we need to trace the image to get a nice clean outline.

- Select the image, then: Bitmaps > Centrline Trace > Line Drawing

- Slide the Detail Slider almost to the end of the + end until you get a clean unbroken outline.

- Slide Smoothing and Corner Smoothness to about 25.

- Click OK.

You might like to ‘play’ with the three sliders until you get an image you like

Step 3.

- Delete the original.

- You will probably need to clean up the image to make sure there are no overhanging lines or errors.

- Use the Virtual Segment Delete tool to do this.

- Zoom in tight!

Step 4.

Set the correct size.

- Select your drawing and then; Object size > 60 x 60

Step 5.

Now we need to make a copy of the outside line so that we can laser cut the base and the top.

- Select your drawing and then; Arrange > Ungroup

- You should be able to select the outside line; Edit > Copy > Edit > Paste

You just need one outside line. We will just cut it twice, once for the top, once for the base.

Step 6.

Last step is to make all the cut lines Hairline in thickness and the lines to be rastered 0.5pt

- Select the lines to be cut then; Pen Tool > Hairline

- Select the lines/drawings to be restored then; Pen Tool > 0.5pt